### Annex II + III

- Technical Specifications and Offer

- ANNEX II + III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: 	Contract title: Water Supply and Sewerage Improvements and Construction of Waste Water Treatment Plant in the Municipality of Drniš

Supply of Operational and Maintenance Equipment

- Publication reference: EuropeAid/130486/M/SUP/HR

- Annex III – the Contractor’s technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 has been completed by the Contracting Authority to show the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

- Column 5 is reserved for the evaluation committee

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

NOTE: Unless otherwise specified, the requirements in these Technical Specifications are presented as a minimum standard which the offered goods must meet.

SEWERAGE MAINTENANCE VEHICLE

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- |
| 1 | Flushing Vehicle 4 m3 with Supplementary Materials – Quantity 1 |  |  |  |
|  | This vehicle shall be provided in order to carry personnel and necessary materials for sewer operation and maintenance activities. |  |  |  |
|  | Chassis |  |  |  |
|  | Type of vehicle: Flatbed truck and chassis 12 t 210 HP 4x2 BB |  |  |  |
|  | Wheelbase: at least 3000 mm |  |  |  |
|  | Steering: left side |  |  |  |
|  | Front-axle load: at least 4,4 t |  |  |  |
|  | Rear axle load: at least 8,4 t |  |  |  |
|  | Permissible gross weight: 11 990 kg |  |  |  |
|  | Front axle and tyres: 2 * 10 R 22,5 |  |  |  |
|  | Rear axle and tyres: 4 * 10 R 22,5 |  |  |  |
|  | Spare wheel and tyre: 1 * 10 R 22,5 |  |  |  |
|  | Colour of chassis: RAL 9005 (jet black) |  |  |  |
|  | Equipment for Vehicle 4 m3 |  |  |  |
|  | Engine, Cooling system and clutch |  |  |  |
|  | Motor 210HP/160 kW, achieving the EURO5 standard without additives, Common Rail |  |  |  |
|  | Torque 650 Nm at rotational speed 1.200-2.100 min-1 |  |  |  |
|  | EDC (Electronic Diesel Control) engine regulation |  |  |  |
|  | Fuel filter with additional separate water trap |  |  |  |
|  | Fuel filter heating |  |  |  |
|  | Control module for external data exchange with body functions |  |  |  |
|  | 1cylinder air compressor |  |  |  |
|  | Exhaust valve brake |  |  |  |
|  | Flame starting system |  |  |  |
|  | Single plate clutch |  |  |  |
|  | Speed limiter electronic 90 km/h and cruise control |  |  |  |
|  | Anti-noise package for 78 dB (92/97 EEC) |  |  |  |
|  | Confirmation for international registration |  |  |  |
|  | Provision for engine start and stop at frame end |  |  |  |
|  | Power Take Off NMV 850/10 |  |  |  |
|  | Output flange |  |  |  |
|  | Transmission Gear Box |  |  |  |
|  | Transmission Gear Box 9 + 1 gears |  |  |  |
|  | Intake/Exhaust System |  |  |  |
|  | Air intake to be raised and with a dry air filter |  |  |  |
|  | The exhaust to be upswept left |  |  |  |
|  | Front Axle/Front Springs/Front Axle Load |  |  |  |
|  | Front axle dropped |  |  |  |
|  | Front springs parabolic 4,4 t |  |  |  |
|  | Stabilizer for front axle |  |  |  |
|  | Rear Axle/Rear Springs/Rear Axle Load |  |  |  |
|  | Rear springs, parabolic 8,4 t |  |  |  |
|  | Hypoid rear axle |  |  |  |
|  | Differential lock in rear axle |  |  |  |
|  | Rear axle breather swept upwards |  |  |  |
|  | Stabilizer for rear axle |  |  |  |
|  | Permissible rear axle load 8,4 t |  |  |  |
|  | Wheels |  |  |  |
|  | Disc wheels 6-hole 6.75x22.5 on 1st front axle |  |  |  |
|  | Disc wheels 6-hole 6.75x22.5 on 1st rear axle |  |  |  |
|  | Disc wheel 6-hole 6.75x22.5 on spare wheel |  |  |  |
|  | Spare-wheel holder on left side |  |  |  |
|  | Fuel Tank |  |  |  |
|  | Fuel tank capacity: 110 l on right hand side with strainer with key |  |  |  |
|  | Steering |  |  |  |
|  | Steering wheel to have height and inclination adjustment |  |  |  |
|  | Steering wheel lock with immobiliser |  |  |  |
|  | Frame |  |  |  |
|  | Synthetic bumper |  |  |  |
|  | Electronic Brake Force Distribution (EBD) |  |  |  |
|  | Anti-lock braking system (ABS) |  |  |  |
|  | Disc brake for front axle |  |  |  |
|  | Disc brake for rear axle |  |  |  |
|  | Asbestos-free brake linings |  |  |  |
|  | Air dryer |  |  |  |
|  | Cab Exterior |  |  |  |
|  | Central locking |  |  |  |
|  | Right and left kerb mirror |  |  |  |
|  | Front mirror on co-driver's side in accordance with Directive 2003/97/EU |  |  |  |
|  | Heated and electrical adjustable rear view mirrors, left and right |  |  |  |
|  | Cab rear wall without windows |  |  |  |
|  | Sun shade in front of windscreen |  |  |  |
|  | Cab Interior |  |  |  |
|  | Driver’s seat with air suspension |  |  |  |
|  | Co-drivers’ bench for 2 persons |  |  |  |
|  | Washable door interior panelling |  |  |  |
|  | Air conditioning system with automatic temperature control without CFC and HCFC |  |  |  |
|  | Fine dust and pollen filter |  |  |  |
|  | Electric door window lifters for driver and co-driver |  |  |  |
|  | Instruments |  |  |  |
|  | Digital calibrated tachograph |  |  |  |
|  | On-board computer |  |  |  |
|  | Display in instrument panel: Language 1 'Croatian' |  |  |  |
|  | Acoustic warning system for engaged reverse gear at frame end |  |  |  |
|  | Lights |  |  |  |
|  | Halogen twin headlights H7 for right-hand traffic |  |  |  |
|  | Electrical System |  |  |  |
|  | Two-tone electric horn |  |  |  |
|  | Batteries: 2 x 12V 110Ah |  |  |  |
|  | Alternator 28 V, 70A |  |  |  |
|  | Standard package of a vehicle |  |  |  |
|  | Operating manual in CROATIAN (3 copies) and ENGLISH (1 copy) |  |  |  |
|  | Antifreeze down to -25 degrees |  |  |  |
|  | Mud-flap front and rear |  |  |  |
|  | Synthetic mudguards, 3-part, with mud flaps |  |  |  |
|  | Safety equipment according to Croatian standards (First-aid kit, chains, fire extinguisher, hazard sign and lamp etc) |  |  |  |
|  | Towing hitch in accordance with Directive 94/20/EC, max. towing weight 2000 kg, max. vertical load 100 kg |  |  |  |
|  | High-Pressure Jetting Installation |  |  |  |
|  | Tank |  |  |  |
|  | Material of tank: Steel S355J2+N (according to EN 10025) or similar |  |  |  |
|  | Volume of tank: 4 m3 |  |  |  |
|  | Cover for filling tank: A manhole cover Ø450 mm for filling and ventilation of tank according to regulations for protection of potable water |  |  |  |
|  | Manual ball valve 4” for tank emptying |  |  |  |
|  | Piston pump jet: 160 bar at 120 l/min |  |  |  |
|  | Diameter of fittings: DN 20 (3/4”) |  |  |  |
|  | Operational requirements: The pump is to be operated from power-take-off switch (PTO) of chassis by cardan shaft and V-belt drive. |  |  |  |
|  | Climbing aid for easy reaching the top manhole on the vehicle, e.g. with fixed steel U-profiles |  |  |  |
|  | Other Modules |  |  |  |
|  | Electric hour counter for high-pressure pump |  |  |  |
|  | Mechanical safety valve for pump protection |  |  |  |
|  | Water lack alarm with visible and acoustic signal |  |  |  |
|  | Powered Hose reel: 100m for ¾” jetting hose hydraulically driven in both directions |  |  |  |
|  | Hose guide: Automatic hose guide synchronically driven |  |  |  |
|  | Operation board: Made from galvanized steel |  |  |  |
|  | Operation board with all control instruments: - Operation time counter - Rpm counter - High pressure gauge - Hydraulic gauge |  |  |  |
|  | Operation board with all following functions: Emergency off jetting pump on – off hose reel open - closed hose reel wind up – wind off hose reel speed control slow - fast hose reel free wheeling on – off motor speed slow - fast |  |  |  |
|  | Tool box: Galvanized steel with lockable doors |  |  |  |
|  | Nozzle box: Open top mounted |  |  |  |
|  | Fenders: Placed over rear wheels with mud flaps |  |  |  |
|  | Pneumatic system: Connected to brake system with shut off valve. |  |  |  |
|  | Hydraulic system: Operating pressure: 160 bar. |  |  |  |
|  | Halogen spot light With swiveable bracket and 2,5m electric cable and socket |  |  |  |
|  | Cable reel: Spring-pull design with 15m electric cable for spot light |  |  |  |
|  | Tail lamp protection: Steel mesh in front of tail lamps |  |  |  |
|  | Nozzles: 2 pcs For 3/4” hose |  |  |  |
|  | Movable aluminium ladder, length of at least 3500 mm, to be fixed on chassis frame |  |  |  |
|  | Varnishing of Superstructure |  |  |  |
|  | Sand-blasting: Of all individual parts of superstructure |  |  |  |
|  | Varnishing: Multi-layer varnishing of all individual parts of superstructure |  |  |  |
|  | Colour: RAL 5015 (sky blue) |  |  |  |
|  | Warning Plates: Reflecting red/white warning plates to DIN 30710 on rear with rear yellow / red warning plates according to regulation ECE 70 |  |  |  |
|  | Training |  |  |  |
|  | Operational training for 10 days for at least 2 persons. The training should be provided in Croatian language. |  |  |  |
|  | Other |  |  |  |
|  | Operating manual in CROATIAN (3 copies) and ENGLISH (1 copy) |  |  |  |

- After-sales services

| 1. Item | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- | --- |
|  | Response time | 4 h response on-site next business day (9 to 17) during three years of warranty (1 year standard warranty + 2 years commercial warranty) |  |  |  |
|  | Repair time | Maximum one month time period in case that major defaults are detected and maximum 10 days period in case of minor repairs during three years of warranty (1 year warranty + 2 years commercial warranty) |  |  |  |
|  | Commercial warranty covering all the supplied equipment and its components | 2 (two) years, beginning from the expiry of the one year standard warranty |  |  |  |

Done at: ………………………,../../..			by (name) ………………………………….

On behalf of …………………………			(tenderer's stamp and signature)