### Annex II + III

- Technical Specifications and Offer

- ANNEX II + III: TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: 	THE SUPPLY OF EQUIPMENT FOR WATER SUPPLY AND SEWERAGE SYSTEM WITH WASTEWATER TREATMENT PLANT FOR SLAVONSKI BROD

- Publication reference: EuropeAid/130057/D/SUP/HR

- Annex III – the Contractor’s technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 has been completed by the Contracting Authority and shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

- Column 5 is reserved for the evaluation committee

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

NOTE: Unless otherwise specified, the requirements in these Technical Specifications are presented as a minimum standard which the offered goods must meet.

- LOT No. 1 SEWERAGE SUCTION/FLUSHING VEHICLES

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- |
| 1 | Combination Suction / Flushing Vehicle 12,5 m3 –Quantity 1 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
|  | Chassis |  |  |  |
|  | Type of vehicle: Flatbedtruck and chassis Min 26 T 6X4 |  |  |  |
|  | Wheelbase: min 4200 mm |  |  |  |
|  | Front axle load: min 9 t |  |  |  |
|  | Rear axle load: min 19 t |  |  |  |
|  | Colour of chassis: RAL 9005 (jet black) |  |  |  |
|  | Engine, Cooling System and clutch |  |  |  |
|  | Motor min 265 KW, achieving the EURO 5 standard |  |  |  |
|  | Torque 1 800 Nm at rotational speed 1.000-1.400 min-1 |  |  |  |
|  | Volume of the engine min 10.500 ccm |  |  |  |
|  | EDC (Electronic Diesel Control) engine regulation |  |  |  |
|  | Fuel filter with additional separate water trap |  |  |  |
|  | Fuel filter heating |  |  |  |
|  | Control module for external data exchange with body functions |  |  |  |
|  | Exhaust valve brake |  |  |  |
|  | Flame starting system |  |  |  |
|  | Single plate clutch |  |  |  |
|  | Speed limiter electronic 85 km/h and cruise control |  |  |  |
|  | Anti noise package to 80dB (92/97EEC) |  |  |  |
|  | Transmission Gear Box |  |  |  |
|  | Transmission Gear Box 16 + 2 gears |  |  |  |
|  | Intake/Exhaust System |  |  |  |
|  | Air intake to be raised and with a dry air filter |  |  |  |
|  | Front Axle/Front Springs/Front Axle Load |  |  |  |
|  | Front axle straight |  |  |  |
|  | Front springs parabolic min 9 t |  |  |  |
|  | Stabilizer for front axle |  |  |  |
|  | Permissible front axle load min 9 t |  |  |  |
|  | Rear Axle/Rear Springs/Rear Axle Load |  |  |  |
|  | Rear air suspension min 2 x 10.5 t |  |  |  |
|  | Planetary axle |  |  |  |
|  | Differential lock in rear axle |  |  |  |
|  | Stabilizer on both rear axles |  |  |  |
|  | Permissible rear axle load min 2 x 10,5t |  |  |  |
|  | Transmission ratios |  |  |  |
|  | Planetary axle ratio i = 4.33 |  |  |  |
|  | Fuel Tank |  |  |  |
|  | Fuel tank capacity: min 300 l with strainer with key |  |  |  |
|  | Electronic Brake Force Distribution (EBD) |  |  |  |
|  | Anti-lock braking system (ABS) |  |  |  |
|  | Disc brake for front axle |  |  |  |
|  | Drum brake for rear axle |  |  |  |
|  | Asbestos-free brake linings |  |  |  |
|  | Air dryer |  |  |  |
|  | Cab Exterior |  |  |  |
|  | Central locking |  |  |  |
|  | Right and left kerb mirror |  |  |  |
|  | Heated and electrical adjustable rear view mirror |  |  |  |
|  | Cab rear wall without windows |  |  |  |
|  | Sun shade in front of windscreen |  |  |  |
|  | Colour of Cab: RAL 6016 (Turquoise green) |  |  |  |
|  | Cab Interior |  |  |  |
|  | Driver’s seat with air suspension |  |  |  |
|  | Co-drivers’ bench for 2 persons |  |  |  |
|  | Air conditioning system with automatic temperature control without CFC and HCFC. |  |  |  |
|  | Fine dust and pollen filter |  |  |  |
|  | Electric door window lifters for driver and co-driver |  |  |  |
|  | Instruments |  |  |  |
|  | Digital calibrated tachograph |  |  |  |
|  | On-board computer |  |  |  |
|  | Display in instrument panel: Language 1 'Croatian' |  |  |  |
|  | Acoustic warning system for engaged reverse gear at frame end |  |  |  |
|  | Lights |  |  |  |
|  | Halogen twin headlights H7 for right-hand traffic |  |  |  |
|  | 2 Halogen revolving beacons yellow on cab roof left and right |  |  |  |
|  | Electrical System |  |  |  |
|  | Two-tone electric horn |  |  |  |
|  | Batteries: min 2 x 12V 155Ah |  |  |  |
|  | Alternator 28 V, 80A, min 2000W |  |  |  |
|  | Standard package of a vehicle |  |  |  |
|  | Antifreeze down to -25 degrees |  |  |  |
|  | Documentation |  |  |  |
|  | Operator's manual in CROATIAN (3 copies) and ENGLISH (1 copy) |  |  |  |
|  | Suction/High-Pressure Jetting Superstructure Installation |  |  |  |
|  | The suction/high-pressure jetting installation shall be provided for hydrodynamic cleaning and removal of blockings in sewer lines and for transportation of non-combustible liquid and sludge |  |  |  |
|  | Tank |  |  |  |
|  | Material of tank: Min quality steel of type S355J2+N (according to EN 10025) or similar |  |  |  |
|  | Volume of tank: min 12 m3 |  |  |  |
|  | Operating pressure of tank: min 0,5 bar |  |  |  |
|  | Plunger for tank: Pneumatic remote-controlled from switch box |  |  |  |
|  | Plunger for tank: Min 3 operating positions (water/sludge) |  |  |  |
|  | Hydraulic-opening overhead with hydraulic operated locker |  |  |  |
|  | Control valve: By hydraulic wedge bolting |  |  |  |
|  | Drainage valve connector: Electro-pneumatically operated min DN 100 valve with wear-resistant shut-off valve |  |  |  |
|  | Suction hose and its carrier hydraulic operated: Min DN 100 placed on top of tank. |  |  |  |
|  | Tank Support: For protection against accidents and during maintenance-/repair jobs below the tank. |  |  |  |
|  | Water filling pipe DN 50 with pneumatic operated valve |  |  |  |
|  | Equipment for Sucking/Blowing Installation |  |  |  |
|  | Vacuum pump: - Type: Rotary-cooled |  |  |  |
|  | - Containing electric counter of operating hours |  |  |  |
|  | - Operation: Electro-pneumatic clutch switch operated from operating board rear at the vehicle. |  |  |  |
|  | - Power: min 90% vacuum |  |  |  |
|  | - Volume with free passage of air min 1300 m³/h |  |  |  |
|  | Safety valve: For pressure limitation of 0,5bar operating pressure |  |  |  |
|  | Equipment For High-Pressure Jetting Installation |  |  |  |
|  | High pressure piston pump for water: - Containing electric counter of operating hours |  |  |  |
|  | - Pump to be switched on by an Electro-pneumatic clutch connected to the pump shaft and operated from the operating board rear at vehicle. |  |  |  |
|  | - Pump protection against overcharge: automatic, pneumatically operated valve with a water lack alarm and optical and acoustic signal at operating board. |  |  |  |
|  | - Cooling device: gear oil for continuous operation of high-pressure pump. |  |  |  |
|  | - Power: min 110 kW |  |  |  |
|  | - Operating pressure: min 200 bar |  |  |  |
|  | - Flow: min 250 l/min |  |  |  |
|  | - Fittings: DN 25 (1"), min 120m |  |  |  |
|  | Climbing aid for easy reaching the top manhole on the vehicle, e.g. with fixed steel U-profiles |  |  |  |
|  | Other Modules |  |  |  |
|  | Hydraulic pump: One-cycle by load sensing control |  |  |  |
|  | Pneumatic system: To be connected to the air brake installation of the chassis. |  |  |  |
|  | Control board: Illuminated, lockable, with all control- and operation instruments including : - emergency off - PTO on – off - motor start – stop - motor speed slow – fast - motor speed idle running - water filling dome open closed - jetting pump on – off - vacuum pump on – off - lower suction device open – closed - suction dome open – closed - locker for piston lock - unlock Control instruments: - rpm counter - vacuum manometer - jetting manometer - hydraulic manometer - air manometer - operation time counters(in drivers cabin) |  |  |  |
|  | Powered Hose reel: Hydraulically driven in both directions, installed across to the traffic direction a large HOSE REEL hydraulically slewable by 180° to the right for taking-up 120m jetting hose DN 25 (1"). |  |  |  |
|  | Hose guide: Automatic hose guide synchronically driven |  |  |  |
|  | Dodge roll: 300mm diameter with turning radius 360° for proper guide of hose |  |  |  |
|  | 2-nd control board: illuminated, lockable, with all control on big hose reel. |  |  |  |
|  | Manual Hose reel: min 60m of ½” jetting hose manually operated for use of spray guns or nozzles |  |  |  |
|  | Scrap box: Tippable open, capacity: min 80 l |  |  |  |
|  | Tool box: lockable, splash-water proof, minimum capacity min 100 litres. |  |  |  |
|  | Halogen spot light: With swiveable bracket and min 2m electric cable and socket |  |  |  |
|  | Cable reel: Spring-pull design with min 15m electric cable for spot light |  |  |  |
|  | Tail lamp protection: Steel mesh in front of tail lamps |  |  |  |
|  | Under ride: Moveable/lockable |  |  |  |
|  | Nozzles: For 1” and ½” hose |  |  |  |
|  | Side boxes along with both sides of superstructure, long hose boxes (length min 3000 mm) |  |  |  |
|  | Large toolbox with two downwards slewable lockable doors (length min 3000 mm) |  |  |  |
|  | Movable aluminium ladder, length of at least 3500 mm, to be fixed on chassis frame |  |  |  |
|  | Varnishing of Superstructure |  |  |  |
|  | Sand-blasting: Of all individual parts of superstructure |  |  |  |
|  | Varnishing: Multi-layer varnishing of all individual parts of superstructure |  |  |  |
|  | Colour: RAL 2011 (Deep orange) |  |  |  |
| 2 | Suction Vehicle 8 m3 – Quantity 1 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
|  | Chassis |  |  |  |
|  | TYPE OF VEHICLE: Flatbedtruck and chassis 18 T 4x2 |  |  |  |
|  | Wheelbase min 3600 mm |  |  |  |
|  | Front axle load: min 8 t |  |  |  |
|  | Rear axle load: min 13 t |  |  |  |
|  | Permissible gross weight: min 18 t |  |  |  |
|  | Colour of chassis: RAL 9005 (jet black) |  |  |  |
|  | Engine, Cooling system and clutch |  |  |  |
|  | Motor min 200 kW achieving the EURO5 standard |  |  |  |
|  | Torque min 1.300 Nm at rotational speed 1000-1400 xy min-1 |  |  |  |
|  | Volume of the engine min 9.000 cm3 |  |  |  |
|  | EDC (Electronic Diesel Control) engine regulation |  |  |  |
|  | Fuel filter with additional water trap |  |  |  |
|  | Fuel filter heating |  |  |  |
|  | Control module for external data exchange with body functions |  |  |  |
|  | Regulated rocker arm brake |  |  |  |
|  | Single-plate clutch |  |  |  |
|  | Speed limiter electronic 85 km/h and cruise control |  |  |  |
|  | Anti-noise package 80 dB (92/97EEC) |  |  |  |
|  | Intake/Exhaust System |  |  |  |
|  | Air intake to be raised and with a dry air filter |  |  |  |
|  | Front Axle/Front Springs/Front Axle Load |  |  |  |
|  | Front axle dropped |  |  |  |
|  | Front springs parabolic min 8.0 t |  |  |  |
|  | Stabilizer for front axle |  |  |  |
|  | Rear Axle/Rear Springs/Rear Axle Load |  |  |  |
|  | Rear air suspension, min 13 t |  |  |  |
|  | Hypoid rear axle |  |  |  |
|  | Differential lock in rear axle |  |  |  |
|  | Stabilizer on rear axle |  |  |  |
|  | Permissible rear axle load min 13 t |  |  |  |
|  | Fuel Tank |  |  |  |
|  | Fuel tank capacity: min 300 l |  |  |  |
|  | Electronic Brake Force Distribution (EBD) |  |  |  |
|  | Anti-lock braking system (ABS) |  |  |  |
|  | Anti-spin regulator (ASR) |  |  |  |
|  | Disc brake for front axle |  |  |  |
|  | Disc brake for rear axle |  |  |  |
|  | Asbestos-free brake linings |  |  |  |
|  | Air dryer |  |  |  |
|  | Cab Exterior |  |  |  |
|  | Central locking |  |  |  |
|  | Right and left kerb mirror |  |  |  |
|  | Heated and electrical adjustable rear view mirror |  |  |  |
|  | Cab rear wall without windows |  |  |  |
|  | Sun shade in front of windscreen |  |  |  |
|  | Colour of Cab: RAL 6016 (Turquoise green) |  |  |  |
|  | Cab Interior |  |  |  |
|  | Driver’s seat with air suspension |  |  |  |
|  | Co-drivers’ bench for 2 persons |  |  |  |
|  | Air conditioning system with automatic temperature control without CFC and HCFC. |  |  |  |
|  | Fine dust and pollen filter |  |  |  |
|  | Electric door window lifters for driver and co-driver |  |  |  |
|  | Instruments |  |  |  |
|  | Digital calibrated tachograph |  |  |  |
|  | On-board computer |  |  |  |
|  | Display in instrument panel: Language 1 'Croatian' |  |  |  |
|  | Display of operating data |  |  |  |
|  | Acoustic warning system for engaged reverse gear at frame end |  |  |  |
|  | Lights |  |  |  |
|  | Halogen twin headlights H7 for right-hand traffic |  |  |  |
|  | 2 Halogen revolving beacons yellow on cab roof left and right |  |  |  |
|  | Electrical System |  |  |  |
|  | Two-tone electric horn |  |  |  |
|  | Batteries: 2 x 12V min 155Ah |  |  |  |
|  | Alternator 28 V, 80A, min 2000W |  |  |  |
|  | Display in instrument panel: Language 1 'Croatian' |  |  |  |
|  | Display of operating data |  |  |  |
|  | Standard package of a vehicle |  |  |  |
|  | Antifreeze down to -25 degrees |  |  |  |
|  | Documentation |  |  |  |
|  | Operator's manual in CROATIAN (3 copies) and ENGLISH (1 copy) |  |  |  |
|  | Suction Installation |  |  |  |
|  | The suction installation shall be provided for suction and transportation of non-combustible liquid and sludge |  |  |  |
|  | Tank 8.000 l |  |  |  |
|  | Material of tank: min quality steel type S355J2+N (according to EN 10025)or similar |  |  |  |
|  | Volume of tank: min 8 m3 - 500 l water/7500 l sludge |  |  |  |
|  | Operating pressure of tank: min 0,5 bar |  |  |  |
|  | Suction/press valve connector: Manual operated DN 100 valve with wear-resistant shut-off valve |  |  |  |
|  | Water filling pipe DN 50 with manual operated valve |  |  |  |
|  | Manhole DN 600 on sludge chamber |  |  |  |
|  | Climbing aid for easy reaching the top manhole on the vehicle, e.g. with fixed steel U-profiles |  |  |  |
|  | Equipment for Sucking/Blowing Installation |  |  |  |
|  | Vacuum pump: - Type: Rotary -cooled |  |  |  |
|  | - Containing electric counter of operating hours |  |  |  |
|  | - Ancillary requirement: cellular radiator, fan, and circulating valve. |  |  |  |
|  | - Power: 90% vacuum with drive power min 25 kW |  |  |  |
|  | - Volume with free passage of air min 900 m³/h |  |  |  |
|  | Ancillary requirement: containing a silencer, oil separator, oil catching device, and drain for removing lubricant and oil deposits from inlet filter |  |  |  |
|  | Safety valve: For pressure limitation of min 0,5bar operating pressure |  |  |  |
|  | Equipment for clear wather installation |  |  |  |
|  | Wather pump: Operating pressure min 10 bar Flow: min 30 l/min Fittings: DN 13 min 40m |  |  |  |
|  | Control board: Illuminated, lockable, with all control- and operation instruments including - emergency off - motor speed slow – fast - motor speed idle running Control instruments: - rpm counter - vacuum manometer - hydraulic manometer - air manometer - operation time counters(in drivers cabin) |  |  |  |
|  | Manual Hose reel: 40m of ½” jetting hose manually operated for use of spray nozzles |  |  |  |
|  | Halogen spot light: With swiveable bracket and 2,5m electric cable and socket |  |  |  |
|  | Cable reel: Spring-pull design with 15m electric cable for spot light |  |  |  |
|  | Side boxes along with both sides of superstructure, long hose boxes of galvanised steel sheet (length 3500 mm) |  |  |  |
|  | Movable aluminium ladder, length of at least 3500 mm, to be fixed on chassis frame |  |  |  |
|  | Varnishing of Superstructure |  |  |  |
|  | Sand-blasting: Of all individual parts of superstructure |  |  |  |
|  | Varnishing: Multi-layer varnishing of all individual parts of superstructure |  |  |  |
|  | Colour: RAL 2011 (Deep orange) |  |  |  |

- Training for Lot no 1

| 1. Item | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- | --- |
|  | Training | Operational training for 10 days for at least for two (2) persons per each vehicle (Items 1 and 2) The training should be provided in Croatian language. |  |  |  |

- After-sales services for Lot no 1

| 1. Item | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- | --- |
|  | Repair time | Maximum one month time period in case that major defaults are detected and maximum 10 days period in case of minor repairs during three years of warranty (1 year warranty + 2 years commercial warranty) |  |  |  |
|  | Response time | 4 h response on-site next business day (9 to 17) during three years of warranty (1 year warranty + 2 years commercial warranty) |  |  |  |
|  | Commercial warranty covering all the supplied equipment and its components | 2 (two) years, beginning from the expiry of the one year warranty |  |  |  |

Done at: ………………………, ../../..			by (name) ………………………………….

On behalf of …………………………			(tenderer's stamp and signature)

- LOT No. 2 CCTV SEWER INSPECTION CAMERA EQUIPMENT INSTALLED IN A CUSTOMIZED VAN

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- |
| 1 | CCTV Sewer inspection Camera Equipment installed in a customized van – Quantity 1 |  |  |  |
|  | Manufacturer’s name: |  |  |  |
|  | Product type, model: |  |  |  |
|  | Mobile TV camera shall be provided, in order to inspect and monitor sewer pipes of diameter 150– 1500 mm for various defects or errors. |  |  |  |
|  | All the devices shall be in exact conformance with EN 13508-2. |  |  |  |
|  | All components shall be installed on an aluminium desk /cupboard and delivered as a combined functional unit. |  |  |  |
|  | Pan and Rotate Camera |  |  |  |
|  | Size: For 150 mm up to 1500 mm diameter pipes |  |  |  |
|  | Settings: Right side up picture with extensive viewing direction programming |  |  |  |
|  | Lighting - LED lighting |  |  |  |
|  | Folding plug, bendable vertically and horizontally |  |  |  |
|  | Tractor guide centraliser for egg profile pipes 200/300, 300/450, 400/600, 600/900, 900/1200, 1000/1500 |  |  |  |
|  | Protection class: IP68 |  |  |  |
|  | Zoom: 10x optical, 4x digital with intelligent auto-focus |  |  |  |
|  | Allowing diameter measurements |  |  |  |
|  | Supplied with a transport case |  |  |  |
|  | Integrated laser for measuring in pipes |  |  |  |
|  | Integrated switchable detector transmitter |  |  |  |
|  | Tractor for Installation of Cameras |  |  |  |
|  | For 150 mm up to 1500 mm diameter pipes |  |  |  |
|  | Steering by Automatic Tilt Compensation |  |  |  |
|  | Electric height-adjusting device DN 250 |  |  |  |
|  | Measurement of diameter with overlay in EDE7 for 300 mm pipes upwards |  |  |  |
|  | Wheel set: 78,93,108, 130 mm |  |  |  |
|  | Pneumatic tyres |  |  |  |
|  | Axle extension for rim 130 mm |  |  |  |
|  | Inclinometer Unit without adjusting device |  |  |  |
|  | Transport handle with bar |  |  |  |
|  | Lowering claw type ASG |  |  |  |
|  | Pressure Test Set for Camera with hand pump and air dehumidifier with pressure gauge |  |  |  |
|  | Ancillary light in LED for pipes 400 mm diameter upwards |  |  |  |
|  | Set of tools for tractor wheels replacement |  |  |  |
|  | Cable |  |  |  |
|  | Length: 300 m |  |  |  |
|  | Connector- waterproof |  |  |  |
|  | Winch |  |  |  |
|  | Motor-driven operated from remote control unit with additional foot control |  |  |  |
|  | Material: Steel |  |  |  |
|  | With boom, steel rope winch |  |  |  |
|  | With workplace light |  |  |  |
|  | Digital counter |  |  |  |
|  | With Cable defection unit to deflect the camera cable |  |  |  |
|  | Power Pack for Mains and Generator Operation with Plug |  |  |  |
|  | Power supply to the workplace light and winch for mains and generator operation with plug |  |  |  |
|  | Power requirements: Input 250 V AC, output 24 V DC 600 W |  |  |  |
|  | Control Unit |  |  |  |
|  | Control panel |  |  |  |
|  | With 2 multifunctional joysticks for operation of the camera and the tractor functions |  |  |  |
|  | For permanent installation in a vehicle in 19“ technology |  |  |  |
|  | Display for operating functions with data display generator and interface |  |  |  |
|  | Connectors: Video in and out with S-Video out |  |  |  |
|  | Operating voltage 240 V AC ± 10 % |  |  |  |
|  | Vehicle mounting kit and power distribution panel |  |  |  |
|  | PC |  |  |  |
|  | Flat screen LCD Monitor 19’’ colour |  |  |  |
|  | Colour LCD Video Monitor 15.1” |  |  |  |
|  | Housing as slide-in unit |  |  |  |
|  | Dual core processor 2,2 GHz |  |  |  |
|  | Graphics card with two-monitor capability |  |  |  |
|  | RAM 2 x 512 MB DDR RAM |  |  |  |
|  | HDD ≥ 160 GB as drive c: |  |  |  |
|  | HDD ≥ 160 GB as drive d: |  |  |  |
|  | DVD ± RW drive burner |  |  |  |
|  | Serial port for EDE and video recorder |  |  |  |
|  | Operating system that supports software for evaluation of CCTV inspection |  |  |  |
|  | Software for image and backup generation |  |  |  |
|  | Keyboard, optical mouse |  |  |  |
|  | MPEG ½ -USB Box (Encoder) with connection cable |  |  |  |
|  | Colour Ink Jet Printer |  |  |  |
|  | With installation package |  |  |  |
|  | Resolution: 4800dpi |  |  |  |
|  | Output: 36 pages/minute |  |  |  |
|  | A3 paper printable |  |  |  |
|  | USB Interface |  |  |  |
|  | USB 2 Hard disk in External Housing, 500 GB |  |  |  |
|  | Uninterrupted power supply for PC system |  |  |  |
|  | Software support |  |  |  |
|  | With Coding system for Norm HR 1503-2 |  |  |  |
|  | Vehicle version including one interface module with corresponding data acquisition and report system |  |  |  |
|  | Length counter switchover |  |  |  |
|  | Defects measurement option |  |  |  |
|  | Diameter measurement option with installation of laser and Interface Card |  |  |  |
|  | Temperature measurement |  |  |  |
|  | Deformation measurement |  |  |  |
|  | Player software for MPEG 2 and MPEG 4 |  |  |  |
|  | 32 bit Software for operating system |  |  |  |
|  | Section graphics in all operating situations with defects entries |  |  |  |
|  | Video overlay with text memory, configurable section heading |  |  |  |
|  | Immediate picture digitisation for pipe sections and manholes |  |  |  |
|  | Live video display on the PC monitor with photo report and installed compatible video PC hardware |  |  |  |
|  | Interrogations for manhole and sewer reports, section graphics, summary data and video tape recording lists |  |  |  |
|  | Report viewer for licence-free display of reports, photos and films in the office |  |  |  |
|  | Management of digital films -MPEG |  |  |  |
|  | Module-overlapping management of several inspection projects |  |  |  |
|  | Project safe, load and append |  |  |  |
|  | Export of inspection data to the corresponding interface |  |  |  |
|  | Language: Croatian |  |  |  |
|  | Tilt measurement report |  |  |  |
|  | Van with high roof |  |  |  |
|  | Type: Box-type VAN with high roof |  |  |  |
|  | Engine type: min 2,5 l (DPF) min 100 kW |  |  |  |
|  | Gearbox: 6 speed manual |  |  |  |
|  | Front stabilizer, up rated |  |  |  |
|  | Rear stabilizer |  |  |  |
|  | Front axle, up rated to 1800 kg |  |  |  |
|  | Up rated front and rear suspension |  |  |  |
|  | Entry grab handle on rear pillar |  |  |  |
|  | Outer rear view mirrors, powered and heated |  |  |  |
|  | Side marker lights |  |  |  |
|  | Warning Markings, red/white |  |  |  |
|  | Power steering with safety steering column |  |  |  |
|  | Air conditioning |  |  |  |
|  | Electric auxiliary heater |  |  |  |
|  | Pivoted seat in cab, left with lumbar support |  |  |  |
|  | Outdoor temperature gauge |  |  |  |
|  | Emergency lantern, warning triangle and first aid kit |  |  |  |
|  | Tool kit and jack |  |  |  |
|  | Battery charger |  |  |  |
|  | Revolving Roof Lights with relay switching and switch on the dashboard |  |  |  |
|  | Colour of the body of the van: RAL 5010 (Gentian Blue) |  |  |  |
|  | Van Interior: Operator’s section |  |  |  |
|  | Insulation of outside walls |  |  |  |
|  | 2 Magnetic boards 1000x60 mm |  |  |  |
|  | Edge protection at side door with step |  |  |  |
|  | Partition with tinted sliding window towards rear section |  |  |  |
|  | Protection wall in vicinity of sliding door |  |  |  |
|  | Angular worktop |  |  |  |
|  | Additional worktop for monitors |  |  |  |
|  | Cabinet mounted with a drawer, a compartment for ISO 216 - A4 files and a partitioned compartment (1“) with a roll-down door |  |  |  |
|  | Wall cabinet on partition towards rear section with 2 roll-down doors |  |  |  |
|  | Emergency light |  |  |  |
|  | Roof hatch 400x400 mm with blind and insect net |  |  |  |
|  | Fluorescent lamp with shade, 230 V AC |  |  |  |
|  | Swivel chair with gliders |  |  |  |
|  | CEE vehicle power inlet with CEE socket, extension cable, adapter and plug type F/CEE coupling |  |  |  |
|  | Van Interior: Rear section |  |  |  |
|  | Lining of side walls and floor |  |  |  |
|  | 3 Aluminium wheel boxes for tractor, tools, accessories and camera |  |  |  |
|  | Winch platform, board lined |  |  |  |
|  | Electric auxiliary heater |  |  |  |
|  | Outside socket 230 V |  |  |  |
|  | Minimum 50 litre water tank under winch platform with filling device and cleaning opening in winch platform |  |  |  |
|  | Damp room lamp, 230 V AC |  |  |  |
|  | Van Interior: Air conditioning and generator |  |  |  |
|  | Installation of a drawer for transport of a generator 4kW with vents in the winch platform and CE adaptor at rear |  |  |  |
|  | Automatic Air conditioner 1,800W with two air outlets partitioned in operator’s section and rear section and one outlet in front of the build-under cabinet |  |  |  |

- Training for Lot no 2

| 1. Item Number | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- | --- |
|  | Training | Operational training for 5 days for at least two (2) persons. The training should be provided in Croatian language. |  |  |  |

- After-sales services for Lot no 2

| 1. Item Number | 2. Specifications Required |  | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes YES/NO |
| --- | --- | --- | --- | --- | --- |
|  | Repair time | Maximum one month time period in case that major defaults are detected and maximum 10 days period in case of minor repairs during three years of warranty (1 year warranty + 2 years commercial warranty) |  |  |  |
|  | Response time | 4 h response on-site next business day (9 to 5) during three years of warranty (1 year warranty + 2 years commercial warranty) |  |  |  |
|  | Commercial warranty covering all the supplied equipment and its components | 2 (two) years, beginning from the expiry of the one year warranty |  |  |  |

Done at: ………………………, ../../..			by (name) ………………………………….

On behalf of …………………………			(tenderer's stamp and signature)